# King Fahd University of Petroleum & Minerals

Civil Engineering Department

# CE317

# Computer Methods in CE

# CE317 : Computer Methods in Civil Engineering

# Lecture 14: Two-Dimensional Unconstrained Optimization:

Our objective is to find the minimum or maximum of a function of two variables say f(x,y). We will extend the Newton-Raphson method used for 1-d case to the present 2-d case. The procedure is explained below:

xi+1yi+1 = xiyi−H−1∂f∂x∂f∂y

where the matrix H is given by:

H = ∂2f∂x2∂2f∂x∂y∂2f∂x∂y∂2f∂y2

So that the above equation becomes:

xi+1yi+1 = xiyi−1∂2f∂x2∂2f∂y2−(∂2f∂xy)2i∂2f∂y2−∂2f∂xy−∂2f∂xy∂2f∂x2i∂f∂x∂f∂yi

In order to check whether the roots make f(x,y) maximum or minimum, we need to investigate second derivatives as follows:

If H> 0 and ∂2f∂x2>0, then f(x,y) has a local minimum.

If H> 0 and ∂2f∂x2<0, then f(x,y) has a local maximum.

If H< 0 , then f(x,y) has a saddle point.

In order to solve equations (1) and (2), we will apply the 2-d Newton-Raphson iterative equation which can be written as:

where all quantities on the right hand side of the above equation is evaluated at xi.

Example 1

Determine the stationary points for the following function and check whether f becomes maximum or minimum at these points.

f(x) = 2 x y +2x -x2 -2y2

Solution:

- Find ∂f∂x,  ∂f∂y, ∂2f∂x2,….

∂f∂x=				∂f∂y=				∂2f∂x2=

∂2f∂y2=				∂2f∂xy=

- Assume the initial guesses for x and y:

x0 = 1, y0 = 1 and calculate x1, y1 using the above iterative equation:

x1y1 = 11−14−4−2−2−22−2 = 21

- Repeat step 2 until convergence. How many iterations do we need for this problem?

- Check whether the solution gives minimum or maximum value of f(x,y). How?

Example 2Determine the stationary points for the following function and check whether f becomes maximum or minimum at these points.

f(x) = x4 + y4+2 x2 y2 -4x +3

Solution:

- Find ∂f∂x,  ∂f∂y, ∂2f∂x2,….

∂f∂x=				∂f∂y=				∂2f∂x2=

∂2f∂y2=				∂2f∂xy=

- Assume the initial guesses for x and y:

x0 = 1, y0 = 1 and calculate x1, y1 using the above iterative equation:

x1y1 = 11−119216−8−81648 = 10.5=

- Repeat step 2 until convergence. After 3 iterations, we get:

x( 3 )= 1.00546     y( 3 )= 0.0107369

x3y3= 1.0050.011

and after 4 iterations, we get:

x4y4= 1.0000.000

- Check whether the solution gives minimum or maximum value of f(x,y). How?

 Important: This handout is only a summery of the lecture. The student should take detailed notes during the class and refer to the textbook for more examples and discussion.