# King Fahd University of Petroleum & Minerals

Civil Engineering Department

# CE317

# Computer Methods in CE

# Lecture 6

# Roots of Single Equations (Continue…)

# Open Methods

# The Fixed-Point Iteration Method:

The method requires one initial guess only.

►Procedure:

Consider the equation f(x) = 0

- Re-write the equation as x = g(x)

- Assume an initial guess for the root = x0 and calculate the first estimate of the root x1 from: x1=g(x0)

- Repeat step 2 several times until convergence is achieved, i.e.

xi+1=g(xi) until Єa < Єs

Convergence Condition: |g(x)| < 1 in the region of interest.

Example1: Use the fixed-point iteration method to estimate the root of f(x)=e-x-x with an accuracy of Єt= 5%. (The exact root is 0.56714329).

Solution:

The above iterative formula becomes

xi+1=e−xi

Performing the iterations, we get:

Iter#			x				Єt(%)

1			e-0 = 1				76.3

2			e-1 = 0.367879			35.1

3			e-0.367879 = 0.692201		22.1

4			e-0.692201 = 0.500473		11.8

5			e-0.500473 = 0.606244		6.89

.			.

10			e-0.571143 = 0.564879		0.399

Example2: Use the fixed-point iteration method to estimate one of the roots of f(x)=x2-2x-3 with an accuracy of Єt= 5%. (The exact roots are: x=-1 and x=3).

Solution:

Alternative (1): Re-write the equation in the form: x=2x+3, so that g(x)=2x+3. Let X0=0. and use the above iterative formula to generate the following results:

Iter # 			    x

1			1.73205

- 2.54246

.			.

.			.

6			2.99413

Note that the method here, converges to the root x=3.

Alternative (2): Re-write the equation in the form: x=3x−2, so that g(x)=3x−2

For x0=0, we get:

Iter # 			        x

1			-1.50000

2			-0.85714

.			.

.			.

5			-1.00549

So that the method converges to the root x = -1.

Alternative (3): Re-write the equation in the form: x=x2−x−3, so that g(x)=x2−x−3. Starting with x0 = 0, we get:

Iter # 		     	        x

1			-3.0000

2			9.00000

3			69.0000

4			4689.00

It is obvious that the method does not converge for the above choice of g(x). Let us repeat the procedure with a different initial guess, say x0=2.9 which is very close to one of the root. We get:

Iter # 			        x

1				2.51000

- 0.79010

.				….

.				….

- 90.6150

- 8117.47

It does not converge! Even if you try x0 = -1.1 which is very close to the other root, you will find out that the method diverges again!! The reason will be clarified later.

Alternative (4): Re-write the equation in the form: x=x2−32, so that g(x)=x2−32. Starting with x0 = 0, we get:

Iter # 		          x

1			-1.50000

2			-0.37500

.			….

14			-0.66217

15			–1.28076

It is obvious that the method diverges for this choice of g(x). The same thing will happen, even if we start with x0 very close to one of the roots!!

Example 3

Can we guarantee the convergence of the fixed-point iteration method if used to solve the equation f(x) =  cos x – 2x for any arbitrary initial guess?

►Advantages

Very easy to program.

# ►Disadvantages

Diverges if the absolute value of the derivative of g(x) with respect to x is greater than 1 for all x in the interval containing the root. (see example 2).

# Open Methods (Continue)

# The Newton-Raphson Metod :

# ►Key-idea:  The method is based on the first order Taylor expansion, i.e.:

# f(xi+1)≈f(xi)+f'(xi) (xi+1−xi)

When we hit the root, f(xi+1) = 0, then:

xi+1≈xi−f(xi)f'(xi)

►Procedure:

- Assume an initial guess for the root = x0 and calculate the first estimate of the root x1 from: x1=x0−f(x0)f'(x0)

- Repeat step 1 several times until convergence is achieved, i.e.

xi+1=xi−f(xi)f'(xi) until Єa < Єs

The procedure is illustrated by the figure shown below.

►Example1: Use Newton-Raphson method to estimate the root of f(x)=e-x-x. Show all details of the iterations. Hint: the root is located between 0 and 1.

Solution:

| Iter | Xi | f(Xi) | f’(Xi) | Xi+1 | ¦εa(%)¦ |
| --- | --- | --- | --- | --- | --- |
| 1 | 0. | 1. | -2. | 0.5 | 100.00 |
| 2 | 0.5 | 0.1065 | -1.6065 | 0.5663 | 11.71 |
| 3 | 0.5663 | 0.0013 | -1.5676 | 0.5671 | 0.15 |
| 4 | 0.5671 | 0.0000 | -1.5676 | 0.5671 | 0.00 |

►Example2: Repeat Example1 starting with x0 = 5.

Solution:

| Iter | Xi | f(Xi) | f’(Xi) | Xi+1 | ¦εa (%)¦ |
| --- | --- | --- | --- | --- | --- |
| 1 | 5. | -4.9933 | -1.0067 | 0.04016 | 12351 |
| 2 | 0.04016 | 0.92048 | -1.9606 | 0.5096 | 92.12 |
| 3 | 0.5096 | 0.0911 | -1.6007 | 0.5665 | 10.04 |
| 4 | 0.5665 | 0.0010 | -1.5675 | 0.5671 | 0.000 |

# Open Methods (Continue)

Secant Method:

The method is used instead of Newton-Raphson method when the derivative of the function is difficult to obtain or if the slope of the function = 0 near the root. The formula of the method is obtained by replacing the exact derivative in Newton-Raphson’s formula by f(xi)−f(xi−1)xi−xi−1 so that

xi+1=xi−xi−xi−1f(xi)−f(xi−1)f(xi)

►Procedure:

- Assume two initial guess x0 and x1 to calculate the first estimate of the root x2 from: x2=x1−x1−x0f(x1)−f(x0)f(x1)

- Repeat step 1 several times until convergence is achieved, i.e.

xi+1=xi−xi−xi−1f(xi)−f(xi−1)f(xi)  until Єa < Єs

►Example3: Repeat Examples 1 using the secant method with the two initial guesses x0 = 0. and x1 = 1.

Solution:

| Iter | Xi-1 | Xi | Xi+1 |
| --- | --- | --- | --- |
| 1 | 1. | 2. |  |
| 2 | 2. |  |  |
| 3 | 0.48714 |  |  |

►Example4: Repeat Examples 3 using the secant method with the two initial guesses x0 = 2. and x1 = 3.

Solution:

| Iter | Xi-1 | Xi | Xi+1 |
| --- | --- | --- | --- |
| 1 | 2. | 3. | 0.2823 |
| 2 | 3. | 0.2823 | 0.6570 |
| . | . | . | . |
| 5 | 0.5719 | 0.5671 | 0.5671 |

# ►Advantages

Both methods are faster to converge than previous methods

# ►Disadvantages

- N-R method diverges if the derivative of x evaluated at the initial guess is zero.

- The secant method requires two initial guesses. However, the guesses do not have to bracket the root.

 Important: This handout is only a summery of the lecture. The student is expected to take detailed notes during the class and refer to the textbook for more examples and discussion.