# CEE 395- Materials for Constructed Facilities

# Civil Engineering Materials

### Aluminum

## Week 12, Lecture 20

Notes:

Some facts about aluminum:

- Most plentiful metal on Earth (8% of the Earth's crust), although primarily found as oxides

- 2-3% of electricity consumed in the U.S. is used in aluminum production.

Recycled aluminum is used widely (25%)

- Pure aluminum is not suitable for structural applications; alloying elements are almost always added

#### Uses for Aluminum

- 25% used for containers

- 20% used for building applications (doors, windows, siding)

- 10% used for electrical conductors

- Balance is used for industrial goods, consumer products, aircraft and vehicles

#### Advantages of Aluminum

- Light weight, has 1/3 density of steel

- Has a high strength to weight ratio

- Surface can be hardened by anodizing/hard coating

- Has good thermal and electrical conductivity

- Has alloys that are weldable

- Will not rust

- Has good formability: die cast, easily machined

- Is nonmagnetic, and nontoxic

#### Aluminum Production

Uses what is called "Bayer" process, developed in 1880's

- Sodium-aluminate leaching process to produce alumina (Al2O3).  The electrolytic process (Hall and Heroult) is used to reduce alumina to pure aluminum

- Production starts with the mining of aluminum ore, bauxite. Bauxite is 45-60% alumina

- Bauxite is crushed, and washed to remove minerals and then dried to remove water

- Bauxite is mixed with soda ash and lime, passed through a digester, pressure reducer, and settling tank to produce a concentrated solution of sodium aluminate (few impurities)

- Solution is seeded with hydrated alumina crystals.

These attract other alumina crystals and settle out of solution

- Crystals are washed to remove remaining impurities and then reduced using the Hall-Heroult process.  Molten aluminum (99% pure) is the result

- Aluminum is placed in furnace to further remove impurities and to add any alloy elements

- Aluminum is either shipped to a foundry for casting or casts into ingot (See table 4.1. for Casting and Forming methods)

### Aluminum

## Week 12, Lecture 20              (Con’t)

Notes:

- Powder metallurgy can be used to produce high strength aluminum.  A powdered aluminum is packed in a mold and heated until it is fused.

#### Aluminum Metallurgy

- Aluminum has a FCC lattice structure that is very malleable.  Typical elongation is over 40%

- Pure aluminum (more than 99% aluminum) is typically used for non-structural applications such as electrical conductors and sheet metal

- Aluminum is typically very weak, although by adding alloys, it can become up to 15 times stronger

- Alloys used for Aluminum - Copper, Manganese, Silicon, Magnesium, and Zinc

- Cold working increases aluminum's strength

- Typical hat treatment for aluminum alloys include annealing, hardening, aging, and stabilizing

#### Alloy Designation (Grading) System

- Wrought aluminum alloys are used for products fabricated by deforming to it's final shape. They are given a 4-digit code

- The first digit is the alloy series

- The second digit indicates a modification to the basic alloy if different from 0

- The third and fourth digits identify the specific alloy used

- Cast aluminum alloys are formulated to flow directly into the mold.  They are given a 30digit code and 1 digit after a decimal point

- The first digit is the alloy series

- The second and third digits identify special alloys

- The digit after the decimal point indicates if the alloy composition is for the final casting

#### Temper Treatments

- Specification of an aluminum includes how it was tempered

- Aluminum alloys for structural applications are classified as either heat treatable or not

- Heat treatment does not influence the mechanical properties

- The properties of these alloys are dependent on the amount of cold working

#### Aluminum Testing and Properties

- Tests are similar to those of steel

- Stress strain curve is linear up to the proportional limit and smooth curve up to the ultimate strength

- No upper and lower yield point

- Elastic modulus (69 GPa or 10,000 ksi). Not affected by alloying or temper treatments

### Aluminum

## Week 12, Lecture 20               (Con’t)

Notes:

- Yield strength (based on 0.2% offset)

- 28 - 255 MPa (Non-heat treatable)

- 55 - 538 MPa (Heat treatable)

- 69 - 276 MPa (Cast)

- Ultimate strength: 69 - 1799 MPa

- Percent elongation: 5% - 43% (1/16 inch)

- Fatigue endurance limit: 21-165 MPa

- Coefficient of thermal expansion is

0.0000023 / C (0.000013 / F).  Twice as large as steel and concrete

- Strength values are considerably affected by temperature

- Temperature above 150 C (300 F), tensile strengths are reduced considerably.  Depends on alloy.

- See Table 4.4 and 4.5 for wrought AL, Table 4.6 for cast aluminum alloys

- Values shown are only indicators.  Material properties of cast members can vary throughout the body of the casting due to differential cooling rates

#### Corrosion

A.  Oxidation

Aluminum will immediately form a layer of oxidation on its surface, which must be penetrated during welding

- The oxidation layer prohibits further oxidation underneath

- Alloys can alter the corrosion resistance of aluminum

- Painting is not necessary in most structural applications

B.  Galvanic Corrosion

Occurs between two dissimilar metals when wet.

Can be prevented by painting the junction, insertin an insulator between the two pieces, or keeping them dry.

#### Welding

Welding requires breaking the tough oxide layer and keeping it from forming again.  Inert gas is used around weld joints to keep oxide from forming during welding, which

would prohibit good weld results

Two common methods:

1.  GMAW - Gas Metal Arc Welding

Filler metal rod serves as one of the two electrodes (the other is clamped to another spot on the item being welded)

2.  GTAW - Gas Tungsten Arc Welding

A tungsten electrode, which does not melt, is used to melt a separate filler rod into the joint.

The heat involved in welding affects the strength of the aluminum within one inch of the weld, decreasing its tensile strength as much as 43% in some cases.

### Aluminum

## Week 12, Lecture 20                (Con’t)

Notes:

#### Fastening with Bolts or Rivets

Bolts and rivets can join aluminum members

1.  Bolts

To prevent galvanic corrosion, they must be aluminum, aluminized steel, galvanized steel, cadmium plated steel, or stainless steel

2.  Rivets

Usually aluminum and cold-driven

Both bolted and riveted joints are designed based on the shear strength of the fasteners and the strength of the members around the fasteners

#### Summary

- Aluminum is not widely used in Civil Engineering applications, mostly because of its expense and lack of performance information, relative to that available for steel products

- Aluminum's advantages include light weight, high strength to weight ratio, and corrosion resistance.

- Alloys and heat treatments can widely alter aluminum's characteristic