Structural Mechanics I

CE 203-KFUPM-021

Summary of lecture 34-37

- Stresses in Thin-walled Pressure Vessels

- Compound Normal and Shear Stresses.

Stresses in Thin-walled Pressure Vessels:

Pressure Vessels: carry internal pressure pint, and resist by developing stresses in the thickness t of the vessel. Thin-walled vessels usually have thickness to radius ratio t/r much less than 10 otherwise it is called a thick-walled pressure vessel. We study here only thin-walled vessels.

Sec. -1

Sec. -2

Types of Vessels: Cylinderical and spherical vessels. Each has an internal radius r, and wall thickness t, as shown in the following two Figures.

Resistance: the vessel resists the effect of internal pressure pint by developing circumferential (hoop) stress h and longitudinal stress l.

Computing Hoop and Longitudinal Stresses:

- Cylinderical vessel: using section 1, the principle of equilibrium between internal force effect Fint, and the resistance developed in the wall thickness R gives:

R = Fint  ===> p r2 = l re2 – ri2] = l re – ri] re + ri]

= l t] [2r] ===> l  = [r/(2t)] p.

And from the section 2:

R = Fint  ===> p l (2r) = h (2 lt] = h l t

===> h  = [r/(t)] p.

Note: h  = 2 l . This means that the hoop stress is more damaging than the longitudinal stress.

- Spherical vessel: using section 1, the principle of equilibrium between internal force effect Fint, and the resistance developed in the wall thickness R, it is easily seen that

R = Fint  ===> p r2 = l re2 – ri2] = l re – ri] re + ri]

= l t] [2r] ===> l  = [r/(2t)] p.

This value is independent of the cut and is easily seen that h  = l for spherical vessels.

Note: The stresses in spherical vessels are much less than in cylinderical vessels. Therefore from a design point of view (that is: selecting appropriate material), spherical vessels should be used to save material for more economical design.

The following example illustrates the applications of the above equations for a typical pressure vessel.

Example: A cylinderical pressure vessel is to be made from a set of wooden pieces shaped together to form a cylinder with internal radius r. A set of steel belts are also used to ensure the integrity of the cylinder under internal pressure as shown in Fig. P-1 below. If the information for the problem are:

- pint = 294 kPa

- r = 1.5 m

- Abelt = 300 mm2

- all   = 100 MPa

- Spacing between belts (unknown) s.

Determine the required spacing of the belts.

Sec. -1

Sec. -2

s

s

Fint

R/2 by the belt

Fig. P-1:

From equilibrium offered by the resistance of one belt (see the FBD above):

Fint = R; then: p (s)(2r) = 2*all (Abelt)

The spacing s = all (Abelt) / (p r)

=  100.E06 (300.E-06)/[ 294.E03 (1.5)] = 0.0680 m

Conclusion:

A proper design will indicate the use of the indicated belts at spacing of not more than 7 cm measured from center to center of each two consecutive belts.

Compound Normal and Shear Stresses:

For a general state of loading there are usually three internal forces (N; Vy; Vz) and three internal moments (Mx = Torsion moment T; My = Bending moment about y-axis; Mz = Bending moment about z-axis that might act on a particular cross section. Each one of theses six internal actions will give a stress of its type to be either normal stress or shear stress The following Fig. 1 illustrates the forces and the stresses in a given point on a given cross section.

yx

zx

x = p

T

Mz

My

N

Vy

Vz

Fig. 1:

Based on the above illustration, the total stress at a point on the cross section defined by coordinate (yp; zp), the total (compont) normal and shear stresses at the point p are computed by linaer superposition from the effects of the individual actions as follows:

- Compound normal stress p =  p (N) p (My)  +p(Mz)

= N/A + My zp/ Iyy + Mz yp/ Izz.

- Compound shear stress p =  p (T)  p (Vy)  p(Vz)

= [Tr/J or T t/Jeq  depending on the type of c/s] + Vy Qzz/ Izz tzz + Vz Qyy/ Iyy tyy

Notes:

- The summation of p is done vectorially as the stresses from the given actions might not be collinear.

- The moments of inertia Iyy and Izz are with respect to the centroidal axes.

- The thicknesses tyy  and tzz are the dimensions for a rectangular c/s or the width of the appropriate cut made on the c/s.

Additional notes and clarifications are included in the following pages.

Applications:

The analysis of compound normal and shear stresses are illustrated by the following examples.

Analysis of Compound Normal and Shear Stresses for a General State of Loading

The 3-D bar shown in the Fig. P-2, is loaded by three forces of 20 kN, 10 kN and 40 kN acting respectively at points A, B, and C. The bar has a rectangle cross section of 20 cm by 15 cm. The cross section at D is fixed to the footing and the three segments of the bar has lengths of 6 m, 4 m, and 5 m.

Determine:

- All internal actions (forces and moments) at section D using vector equilibrium equations.

- The maximum compound normal stress at c/s D.

- The maximum compound shear stress at c/s D.

A

Coordinates of points:

A( 5, 4, -6) m.

B( 5, 4, 0) m.

C( 5, 0, 0) m.

D(0,0,0) m.

20 kN

10 kN

40 kN

5 m

4 m

B

C

x

y

z

D

6 m

corner 1

Fig. P-2:

F1

F2

F3

r2

r3

r1

FD

MD

FBD of center line of the bar ABCD with forces vectors  Fi and position vectors  ri.

Vector Equilibrium equations:

Fext + Fint = 0; Mext. @ point D + Mint. @ point D = 0;

F1 + F2 + F3 = 0; -20 j + 10 i – 40 k + FD = 0. Then: FD = - 10 i  + 20 j + 40 k  (kN).

Mext. @ point D + Mint. @ point D = 0;

(r1x F1) + (r2x F2) + (r3x F3) + MD = 0  … (2)

And with the origin of the (x,y,z) system at point D, the position vectors are:

r1 = 5 i+ 4j-6k (m)

r2 = 5 i+ 4j (m)

r3 = 5 i (m)

Then from equation (2) above, the moment MD is obtained from the sum of the negatives of the cross product of the position vectors ri and forces Fi.

MD =  - [(5 i+ 4j-6k) x (-20j) + (5 i+ 4j) x (10i) + (5 i) x (-40k)]

= - [ (-100k + 0 – 120i)   + ( 0 - 40k) + (200j)].

Interpretations of FD and MD:

The components of the two vector internal actions (forces and moments) are as follows:

FDx = N = -10 kN  (Tension as it is in the –ve x-direction!! ).

FDy = Vy = 20 kN.

FDz = Vz = 40 kN.

MDx = Mx = T = + 120 kN.m.

MDy = - 200 kN.m.

MDz = +140 kN.m.

Internal Actions Placed on the C/S:

10 kN

140 kN.m

200 kN.m

corner 1

The normal stresses and shear stresses at c/s D will be caused by the compound effects of the above internal actions acting on the c/s at D.



The computations of max requires similar procedure of placing the actions T, Vy, and Vz on the c/s and determining the point that will include addition (at point 2) of component shear stresses.

x

z

y

x

tot =  Vy)  + (Vz)

b)(2a)2)]+ VyQzz/Izz tzz + 0. (@ level of  point 2)

where: Qzz = [0.15(0.10)*(0.10/2)] m3; and tzz = 0.15 m.

Mid-length of long side (point 2).

- Stress cube at point 2: with and

tot

tot

complete the computations of the numerical values of the compound stresses!