# University Diploma Programs

# Electronic Equipment Maintenance

##### EET 029, Introduction to Communication

MAJOR EXAM # 1

Date: 4th November 2003

Instructor: M. Ajmal Khan, Lecturer, EE Dept.

Student’s Name : ____________________   ID # : _________

Time Allowed: 2 Hours

| Question # | Points | Score |
| --- | --- | --- |
| 1 | 22 |  |
| 2 | 5 |  |
| 3 | 3 |  |
| 4 | 10 |  |
| 5 | 10 |  |
| 6 | 10 |  |
| 7 | 5 |  |
| 8 | 5 |  |
| 9 | 10 |  |
| Total | 80 |  |

Question # 1: Circle the correct Answer for each of the following questions:

- The SNR is ________________________ along the length of the channel.

  - Increasing

  - Constant

  - Decreasing

  - None of the above.

- The signal level ____________________ and the noise level increases with distance from the transmitter.

  - Increases

  - Constant

  - None of the above.

- In _________________ systems, there are ways to avoid accumulation of noise and distortion along the path.

  - Digital

  - Analog

  - Combined analog and digital

  - None of the above.

- The two primary communication resources are:

  - Channel bandwidth and channel noise

  - Channel bandwidth and transmitted power

  - Transmitted power and noise

  - Transmitted power and modulation

- The process of Amplitude Modulation shifts the spectrum of the modulating signal to ______________________ by .

  - Either left or right.

  - The left only

  - The right only

  - The left and the right

- In DSB modulation, the modulated signal spectrum is composed of :

  - three parts; USB, LSB and Carrier

  - two parts; USB and LSB

  - three parts; USB, LSB and reduced carrier.

  - None of the above.

- The balanced modulator is used:

  - to add full carrier

  - to add a reduced carrier

  - to suppress carrier.

  - None of the above.

- In a communication systems, noise is most likely to affect the signal:

- At the transmitter

- In the channel

- In the information source

- At the destination

- C3F modulation is normally used for:

  - HF point-to-point communications

  - Monaural broadcasting

  - TV broadcasting

  - Stereo broadcasting.

- If the carrier of a 100 percent modulated AM wave is suppressed, the percentage power saving will be:

  - 50

  - 150

  - 100

  - 66.66

- R3E modulation is sometimes used to:

  - Allow the receiver to have a frequency synthesizer

  - Simplify the frequency stability problem in reception

  - Reduce the power that must be transmitted

  - Reduce the bandwidth required for transmission

- To provide two or more voice circuits with the same carrier, it is necessary to use:

  - ISB

  - Carrier reinsertion

  - SSB with pilot carrier

  - Lincompex

- The modulation index of an AM wave is changed from 0 to 1. The transmitted power is:

  - Unchanged

  - Halved

  - Doubled

  - Increased by 50 percent

- Amplitude modulation is used for broadcasting because:

  - It is more noise immune than other modulation systems

  - Compared with other systems it requires less transmitting power

  - Its use avoids receiver complexity

  - No other modulation system can provide the necessary bandwidth for high fidelity.

- The most commonly used filters in SSB generation are:

  - Mechanical

  - RC

  - LC

  - Low-pass

- Indicate in which one of the following only one sideband is transmitted:

  - H3E

  - A3E

  - B8E

  - C3F

- One of the following cannot be used to remove the unwanted sideband in SSB:

  - Filter system

  - Phase-shift method

  - Balanced modulator

  - Third method.

- In the spectrum of a frequency-modulated wave:

  - The carrier frequency disappears when the modulation index is large.

  - The amplitude of any sidebands depends on the modulation index.

  - The total number of sidebands depends on the modulation index.

  - The carrier frequency cannot disappear.

- The difference between phase and frequency modulation:

  - Is purely theoretical because they are the same in practice

  - Is too great to make the two systems compatible

  - Lies in the poorer audio response of phase modulation

  - Lies in the different definitions of the modulation index.

- A pre-emphasis circuit provides extra noise immunity by:

  - Boosting the bass frequencies

  - Amplifying the higher audio frequencies

  - Preamplifying the whole audio band

  - Converting the phase modulation to FM.

- When the modulating frequency is doubled, the modulation index is halved, and the modulating voltage remains constant. The modulation system is:

  - Amplitude modulation

  - Frequency modulation

  - Phase modulation

  - None of the above.

- Indicate which one of the following is not an advantage of FM over AM:

  - Better noise immunity is provided

  - Lower bandwidth is required

  - The transmitted power is more useful

  - Less modulating power is required.

Question # 2: Circle the correct Answers (Check all that Apply) for each of the following questions:

- Channel acts as a filter:

  - To add noise in the signal

  - To attenuate the signal

  - To modulate the input signal

  - To demodulate the signal

  - To distort the signal’s waveform

- Noise is undesirable signals, which are:

  - Random

  - Predictable

  - Uniform

  - Unpredictable

- The performance (accuracy) of analog-to-digital conversion system can be increased.

    - By increasing the number of quantization levels.

    - At sampling rate less than Nyquist rate.

    - By decreasing the number of quantization levels.

    - At sampling rate greater than Nyquist rate.

- If the signal power increases then

- The channel bandwidth must increase.

- One may reduce the channel bandwidth

- The effect of channel noise reduces

- The effect of channel noise increases.

- In DSB modulation, which of the following exists:

- a portion that lies above  is known as USB.

- a portion that lies below  is known as LSB.

- a portion that lies below  is known as USB.

- a portion that lies above  is known as LSB.

Question # 3:  Indicate the False statement for each of the following questions:

- Modulation is used to:

  - Reduce the bandwidth used

  - Separate differing transmissions

  - Ensure that intelligence may be transmitted over long distances

  - Allow the use of practicable antennas

- Indicate the false statement regarding the advantages of SSB over double sideband, full carrier AM.

- More channel space is available

- Transmitter circuit must be more stable, giving better reception

- The signal is more noise-resistant

- Much less power is required for the same signal bandwidth

- Indicate which one of the following advantages of the phase cancellation method of obtaining SSB over the filter method is false:

  - Switching from one sideband to the other is simpler

  - It is possible to generate SSB at any frequency

  - SSB with lower audio frequencies present can be generated

  - There are more balanced modulators; therefore the carrier is suppressed better.

Question # 4:    Write the definitions of the following:

- Sampling Theorem

- Quantization.

- Channel Bandwidth.

- Synchronous detection (coherent detection)

- Modulation index

Question # 5:   A 25 MHz carrier is modulated by a 400 Hz audio sine wave. If the carrier voltage is 4 V and the maximum deviation is 10 kHz, write the equation of this modulated wave for (a) FM and (b) PM. If the modulating frequency is now changed to 2 kHz, all else remaining constant, write a new equation for (c) FM and (d) PM.

Question # 6:   Calculate the percentage power saving in an AM wave modulated to a depth of 50 percent, when (a) the carrier is suppressed and       (b) carrier and one of the sidebands are suppressed.

Question # 7:  A 1000 kHz carrier is simultaneously modulated with 300 Hz, 800 Hz and 2 kHz audio sine waves. What will be the frequencies present in the output.

Question # 8:  A broadcast AM transmitter radiates 50 kW of carrier power. What will be the radiated power at 85 percent modulation?

Question # 9:  A 400 W carrier is modulated on a depth of 75 percent, calculate the total power in the modulated wave in the following forms of AM:

- Double-sideband with full carrier

- Double-sideband, suppressed carrier

- Single-sideband suppressed carrier.