# COE 561, Term 111

# Digital System Design and Synthesis

# HW# 2   Solution

# Consider the function F(A,B,C,D) with the following ON-set and DC-set:

# FON= m(0, 2, 3, 4, 5, 7,  8, 10, 12, 13, 15)

# FDC= m(1, 11)

## Apply the EXPAND procedure on the given cover using Espresso heuristics and show the obtained expanded cover. Compare your solution with the result obtained by ESPRESSO tool.

# Consider the function F(A, B, C, D) with ON-SET=m(0, 4, 5, 7, 8, 12, 13, 15) and DC-SET=m(1, 3, 9, 14).

## A cover of the function is given by F = C’ + BD. Reduce the cube C’ using Theorem 7.4.1.

## Use Corollary 7.4.1 to check if the implicant BD is an essential prime implicant.

# Consider the following cover of a function F(A,B,C,D)

## Determine the relatively essential set of cubes, Er.

## Determine the totally redundant, Rt, and partially redundant, Rp, sets of cubes.

## Find a subset of Rp that, together with Er, covers the function by solving a covering problem.

## Compare your solution with the result obtained by ESPRESSO tool.

# espresso -d -t -Dexpand hw2q1.pla

# UC Berkeley, Espresso Version #2.3, Release date 01/31/88

.olb y

# READ       	Time was 0.00 sec, cost is c=11(11) in=44 out=11 tot=55

# COMPL      	Time was 0.00 sec, cost is c=2(2) in=7 out=2 tot=9

# PLA is hw2q1.pla with 4 inputs and 1 outputs

# ON-set cost is  c=11(11) in=44 out=11 tot=55

# OFF-set cost is c=2(2) in=7 out=2 tot=9

# DC-set cost is  c=2(2) in=8 out=2 tot=10

EXPAND: 1010 1 (covered 3)

EXPAND: 0011 1 (covered 2)

EXPAND: 1101 1 (covered 3)

# EXPAND     	Time was 0.00 sec, cost is c=3(0) in=6 out=3 tot=9

# READ       	 1 call(s) for 0.00 sec ( 0.0%)

# COMPL      	 1 call(s) for 0.00 sec ( 0.0%)

# EXPAND     	 1 call(s) for 0.00 sec ( 0.0%)

# expand	Time was 0.00 sec, cost is c=3(0) in=6 out=3 tot=9

.i 4

.o 1

.ilb a b c d

.p 3

-0-0 1

--11 1

-10- 1

.e

# WRITE      	Time was 0.00 sec, cost is c=3(0) in=6 out=3 tot=9

# espresso -Dirred -t -d hw2q3.pla

# UC Berkeley, Espresso Version #2.3, Release date 01/31/88

.olb y

# READ       	Time was 0.00 sec, cost is c=6(6) in=15 out=6 tot=21

# COMPL      	Time was 0.00 sec, cost is c=0(0) in=0 out=0 tot=0

# PLA is hw2q3.pla with 4 inputs and 1 outputs

# ON-set cost is  c=6(6) in=15 out=6 tot=21

# OFF-set cost is c=0(0) in=0 out=0 tot=0

# DC-set cost is  c=0(0) in=0 out=0 tot=0

# IRRED: F=6 E=2 R=4 Rt=1 Rp=3 Rc=1 Final=3 Bound=0

# IRRED      	Time was 0.00 sec, cost is c=3(3) in=7 out=3 tot=10

# READ       	 1 call(s) for 0.00 sec ( 0.0%)

# COMPL      	 1 call(s) for 0.00 sec ( 0.0%)

# IRRED      	 1 call(s) for 0.00 sec ( 0.0%)

# irred	Time was 0.00 sec, cost is c=3(3) in=7 out=3 tot=10

.i 4

.o 1

.ilb a b c d

.p 3

0--1 1

-0-0 1

111- 1

.e

# WRITE      	Time was 0.00 sec, cost is c=3(3) in=7 out=3 tot=10