###### Appendix A- Physical-Layer Media Specifications

| Specifications | 10Base5 | 10Base2 | 10Base-T | 100Base-TX (Fast Ethernet) | 1000Base-TX (Gigabit Ethernet) | FDDI |
| --- | --- | --- | --- | --- | --- | --- |
| Cable Type | Thick coaxial cable | Thin coaxial cable | Twisted-pair | Twisted-pair (two pairs) | Twisted-pair (four pairs) | Fiber optic |
| Cable Impedance | 50 ohms | 50 ohms | 85-110 ohms | 85-110 ohms | 85-110 ohms | - |
| Signaling techniques | Baseband (Manchester) | Baseband (Manchester) | Baseband (Manchester) | Baseband (Manchester) | Baseband (Manchester) | Baseband |
| Data Rate | 10 Mbps | 10 Mbps | 10 Mbps | 100 Mbps | 1000 Mbps | 1000 Mbps |
| Maximum cable segment length | 500 m | 185 m | 100 m | 100 m | 100 m | ~ 2 km |
| Maximum network length | 2500 m | 925 m | 2500 m | 2500 m | 2500 m | 10 km |
| Attachments per segment | 100 | 30 (29 with a repeater) | 1 | 1 | 1 | - |
| Attachments spacing | 2.5 m | 0.5 m |  |  |  | - |
| Connector type | DB-15 connector | BNC | RJ 45 (8 pin) | RJ 45 (8 pin) | RJ 45 (8 pin) | MIC, ST |
| Topology | Linear Bus | Linear Bus | Star-wired bus | Star-wired bus | Star-wired bus | Ring |
| Maximum number of stations per network | 1024 | 1024 | 1024 | 1024 | 1024 | 1000 |