KING FAHD UNIVERSITY OF PETROLEUM AND MINERALS

Electrical Engineering Department
EE-303 Electronics II (072) 

Design Project  
Dr. Hussain Alzaher

Due Date: Saturday May 28, 2008 (In class)

	Name:                                                                  I.D#.                           No.               Sec.  1


The main purpose of this project is to applied what you studied in this course to design  a current amplifier. It is required to complete the following tasks:
(a) Design a discrete transistor feedback current amplifier to provide a current gain of 30. Use only two npn transistors having parameters hFE=βo= 120 and VA=
[image: image1.wmf]¥

. The supply voltage is 10V and use any number of resistors with maximum value of 100kΩ for each.
The signal current source has a source resistance of Rs=25kΩ and the load is RL=500Ω

(b) Determine Rif and Rof.

(c) Verify the design with a computer simulation. 

_1272094423.unknown

