King Fahd University of Petroleum and Minerals

Electrical Engineering Department
EE303 Electronics II 

Quiz # 3
	Name:                                                      I.D#                             No.                       Sec.# 01


(a) It is required use an opamp to design a voltage amplifier such that its gain is 100V/V and bandwidth is 100kHz. What is the required unity gain frequency (ωt) of the opamp. 
(b) If the maximum output of the amplifier in part (a) is 2V, what should be the slew rate of opamp.



