Stat-319

Quiz 4                                                                                                                        Sec: 03

Name:                                                                         ID#:                                          S.No.

A machine produces metal rods used in an automobile suspension system. A random sample of 10 rods is selected, and the diameter is measured. The resulting data (in millimeters) are as follows:

8.24      8.25      8.20      8.23      8.24

8.21      8.26      8.19      8.28      8.23

- Find the mean and median, based on these values is the data skewed positively or negatively? Reason?

- Calculate the sum of the squared deviations and use this value to calculate the standard deviation.

- Find the co-efficient of variation and interpret your result.

- Check the 2nd condition of the Empirical Rule for the above data and comment?

- Find the lower and upper quartiles and use these values to construct a box plot.

- Find a 98% two-sided confidence interval on mean rod diameter.

- Is there evidence that the true mean diameter is not 8.24 mm? Use 0.02 level of significance.

- What is the P-value for this test?

- What will be the value of the test Statistic to check if the mean diameter is 8.24 mm. Assuming the over all standard deviation 0.02 mm.

- What should be the sample size in order to be 98 % confident that the true mean does not differ from the sample mean by more than 0.12 mm.