COE 405

VHDL

By Dr.

Aiman El-Maleh

Rayan Jamal Mufti

985023

Burhan Ali

987528

Q.1.

(i)

The intity description of the four bit carry look ahead adder is as follow:

-----------------------------------------------------------------------------------

--  I N T I T Y   D E S C R I P T I O N  O F  C L A

-----------------------------------------------------------------------------------

--

ENTITY CLA is

PORT (	A,B: 	in 	Bit_Vector (3 DOWNTO 0);	-- A & B ==> four bit inputs each.

C0: 	in 	Bit;			 		-- C0 ==> Carry in (C zero).

S:	out	Bit_Vector (3 DOWNTO 0);	-- S  ==> four bit output.

C4:	out	Bit);					-- C4 ==> Carry out.

END CLA;

--

Q.1.

(ii)

------------------------------------------------------------------------------------

--  A R C H I T E C T U R E  D E S C R I P T I O N  OF  C L A

------------------------------------------------------------------------------------

--

ARCHITECTURE	Arch1 of CLA4 is

SIGNAL C1,C2,C3: Bit;

SIGNAL P,G: Bit_Vector (3 DOWNTO 0);

BEGIN

P(0)	<= A(0)XOR B(0);			-- The propagate function of A & B.

G(0)	<= A(0)AND B(0);  		-- The generate function of A & B.

S(0)	<= P(0)XOR C0 after 5 ns;	-- The sum of A & B.

C1	<= G(0)OR				-- The carry is generated after 2 gates only.

(P(0)AND C0) after 3 ns;

------------------------------------------------------------------------------------

P(1)	<= A(1)XOR B(1);			-- The propagate function of A & B.

G(1)	<= A(1)AND B(1);  		-- The generate function of A & B.

S(1)	<= P(1)XOR C1 after 5 ns;	-- The sum of A & B.

C2	<= G(1)OR(P(1)AND G(0))OR	-- The carry is generated after 2 gates only.

(P(1)AND P(0)AND C0) after 3 ns;

------------------------------------------------------------------------------------

P(2)	<= A(2)XOR B(2);			-- The propagate function of A & B.

G(2)	<= A(2)AND B(2);  		-- The generate function of A & B.

S(2)	<= P(2)XOR C2 after 5 ns;	-- The sum of A & B.

C3	<= G(2)OR (P(2)AND G(1))OR

(P(2)AND P(1)AND G(0))OR    -- The carry is generated after 2 gates only.

(P(2)AND P(1)AND P(0)AND C0) after 4 ns;

------------------------------------------------------------------------------------

P(3)	<= A(3)XOR B(3);			-- The propagate function of A & B.

G(3)	<= A(3)AND B(3);  		-- The generate function of A & B.

S(3)	<= P(3)XOR C3 after 5 ns;	-- The sum of A & B.

C4	<= G(3)OR (P(3)AND G(2))OR

(P(3)AND P(2)AND G(1))OR	-- The carry is generated after 2 gates only.

(P(3)AND P(2)AND P(1)AND G(0))OR

(P(3)AND P(2)AND P(1)AND P(0)AND C0) after 5 ns;

END Arch1;

Q.1.

(iii)

The simulation is working properly.

Example: 	A: 1111

B: 1111 +

C0:   1 +

-------

S: 1111

C4	1

>force A 1111 2ns

>force B 1111 2ns

>force C0   1 2ns

>run 15

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Example:	A: 0101

B: 1010 +

C0:   1 +

-------

S: 0000

C4	1

>force A 0101 2ns

>force B 1010 2ns

>force C0   1 2ns

>run 15

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Example:	A: 1001

B: 1011 +

C0:   0 +

-------

S: 0100

C4	1

>force A 1001 2ns

>force B 1011 2ns

>force C0   0

>run 15

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Q.2.

(i)

The Adder8 uses 2 instants of the four bits CLA. The carry goes out from CLA1 as C4 and connects to CLA2 C4 via an internal signal called “Inter_Carry”.

-----------------------------------------------------------------------------------

--  I N T I T Y   D E S C R I P T I O N  O F  A d d e r 8

-----------------------------------------------------------------------------------

--

ENTITY Adder8 IS

PORT (

SIGNAL X,Y : IN  bit_vector (7 DOWNTO 0);		-- X & Y ==> eight bit inputs each.

SIGNAL Z0  : IN  bit ;					-- Carry in

SIGNAL P   : OUT bit_vector (7 DOWNTO 0) ;	-- P ==> eight bit inputs each.

SIGNAL R8  : OUT bit ) ;				-- Carry out

END Adder8 ;

--

Q.2.

(ii)

--

------------------------------------------------------------------------------------

--  A R C H I T E C T U R E  D E S C R I P T I O N  O F  A d d e r 8

------------------------------------------------------------------------------------

--

ARCHITECTURE Struct OF Adder8 IS

SIGNAL Inter_Carry : bit;

COMPONENT CLA4

PORT 	(

A,B: 	in 	Bit_Vector (3 downto 0);

C0: 	in 	Bit;

S:	out	Bit_Vector (3 downto 0);

C4:	out	Bit

);

END COMPONENT;

BEGIN

-- Instantiation:

CLA1:	CLA4 PORT MAP (	X(3 DOWNTO 0)			--X(3),X(2),X(1),X(0)

Y(3 DOWNTO 0),			--Y(3),Y(2),Y(1),Y(0)

Z0,

P(3 DOWNTO 0),			--P(3),(2),P(1),P(0)

Inter_Carry

);

CLA2:	CLA4 PORT MAP (	X(7 DOWNTO 4), 			--X(7),X(6),X(5),X(4)

Y(7 DOWNTO 4),			--Y(7),Y(6),Y(5),Y(4)

Inter_Carry,

P(7 DOWNTO 4),			--P(7),P(6),P(5),P(4)

C8

);

END Struct;

Q.2.

(iii)

The simulation is working properly.

Example: 	X: 1111 1111

Y: 1111 1111 +

Z0:        1 +

------------

P: 1111 1111

C8	     1

>force X 11111111 2ns

>force Y 11111111 2ns

>force Z0       1 2ns

>run 20

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Example: 	X: 1010 1010

Y: 0101 0101 +

Z0:        1 +

------------

P: 0000 0000

C8	     1

>force X 10101010 2ns

>force Y 01010101 2ns

>force Z0       1 2ns

>run 20

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

Example: 	X: 1110 0111

Y: 1001 1100 +

Z0:        0 +

------------

P: 1000 0011

C8	     1

>force X 11100111 2ns

>force Y 10011100 2ns

>force Z0       1 2ns

>run 20