GEOP 315

Fall 2002

Homework assignment # 9

Stacking Charts

(Due date: January 3, 2004)

Objective

To be able to read stacking charts and use them to setup the geometry and sort the traces into CMP gathers.

Introduction

Stacking charts are used to setup the survey geometry and sort the traces into CMP gathers in preparation for further processing.  The charts are prepared by the field geophysicist and provided to the processor.

# Exercises

Using the provided stacking chart, answer the following questions:

- What is the total number of stations that the operator is using?

- What is the total number of shots that the operator is using?

- What is the number of receivers that the operator is activating with each shot?

- What is the type of spread being used with each shot?

- What is the CMP spacing?

- In tables, sort the traces in:

- CMP domain, indicating fold for each CMP.

- Offset domain, indicating fold for each offset.

1	 2	 3	 4	 5	6	7	8	9	10

A          O	X	X	X	X	X	X	X	X	X

B	X	X	O	X	X	X	X	X	X	X

C	X	X	X	O	X	X	X	X	X	X

D	X	X	X	X	X	O	X	X	X	X

E	X	X	X	X	X	X	X	X	X	O

Spacing between stations = 50 m.

O stands for shot location.

X stands for receiver location.