King Fahd University of Petroleum and Minerals

Department of Computer Engineering

COMPUTER ARCHITECTURE COE 301

Homework 4

To be submitted on 30 December 2014

| Problems | Grading |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| TOTAL |  |

QUESTION 1: REDESIGNING INSTRUCTION SET ARCHITECTURE

A program, being executed on a processor, has its instruction distribution as: (1) ALU takes 2 cycles (40%), (2) Load takes 5 cycles (20%), Store takes 4 cycles (15%), and Branch takes 3 cycles (25%). Answer the following independent questions:

a)	Compute the average clock cycles per instruction (CPI).

b)	To improve performance the designer designed a new ALU so that 80% of its operations take 1 cycle and the other 20% still take 2 cycles. Compute the new CPI.

c)	Another alternative to improve performance is to redesign the Load and Store Units so that a load instruction takes 2 cycles in 95% of the cases and 10 cycles in the remaining 5%. The Store Unit is also improved so that each store takes 1 cycle to execute. Compute the new CPI.

d)   Evaluate the CPI for the Combined Design Improvements (CDI) specified in questions b and c?

e)	The designer decides to improve the clock speed of the CDI in such a way to reduce overall average execution time to half (1/2) of the original CPU specified in part a. By what factor (ratio of clock rates) should the clock speed be improved?

QUESTION-2: PRINCIPLES OF HIERARCHICAL MEMORY

The idea of caching code and data is based upon the principle of locality. Give a (1) short definition and (2) one example for each type of locality:

- Give a short definition and an example of temporal data locality

- Give a short definition and an example of temporal code locality

- Give a short definition and an example of spatial data locality

- Give a short definition and an example of spatial code locality

QUESTION-3: HIERARCHICAL MEMORY SYSTEMS

A computer has a processor P with a hierarchical memory system that consists of a 4-ways set-associative cache memory CM and  a main-memory MM. P generates a 32-bit memory address ADDR = (TAG,  Index, Word). CM has 64-entries. The page size is 32 bytes, where each word addressed by P is one byte.  MM has Tmm=18 ns access time. Denote by (Tcm) and (Ph) as CM access time and its hit probability, respectively.

Answer each of the following questions:

- Find the number of bits in “Index”, “Word”, and “TAG”.

- Determine the page size in bits.

- Determine the cache capacity in bits.

- Determine the main memory capacity in bits.

- CM can be designed in the following ways: (1) as a 4-way set-associative cache with Tcm = 1.25 ns and Ph = 0.88, (2) as a fully-associative cache with Tcm = 1.5 ns and Ph = 0.97, (3) as a direct-mapped cache with Tcm = 1 ns and Ph = 0.80. Answer each of the above questions:

  - Which of the above three cache designs has the least conflict misses.

  - For what reason the direct-mapped cache has the least access time.

  - For what reason the cache with largest hit probability has the largest access time.

  - Determine which of the above three cache designs gives the best global performance of hierarchical memory system including CM and MM.

- The processor is running a benchmark with the following instruction distribution: (1) 55% of R-type, (2) 18% are loads, (3) 12% are stores, and (4) 15% are branching instructions. Suppose the retained cache solution is as a 4-way set-associative cache with Tcm = 1.25 ns and Ph = 0.8 which is used as an instruction cache and as a data cache. Answer each of the following questions:

  - Evaluate the average access time of the memory.

  - Evaluate the average stall time per instruction (Tstall).

  - Evaluate the CPI for this processor if the ideal CPI is 2 clocks and processor clock rate is 1 GHz.