KING FAHD UNIVERSITY OF PETROLEUM & MINERALS

DEPARTMENT OF ELECTRICAL ENGINEERING

EE 417:   Communications Engineering II

Major Exam I

| # NAME |  |
| --- | --- |
| # ID |  |

| ##### Question | ##### Score |
| --- | --- |

| 1 |  |
| --- | --- |
| 2 |  |
| 3 |  |

| ##### TOTAL |  |
| --- | --- |

Problem #1 (10pts)

The PSD of a Gaussian noise process N(t) is given by SN(f)= rect[(f-fc)/2B]+ rect[(f+fc)/2B],  (expressed in W/Hz), where fc is a high frequency value and B is a constant bandwidth (<fc).

- What type of noise is this? Write its in-phase/quadrature representation (using NI(t) and NQ(t)).

- What is the total average power contained in N(t)?

- Prove that the autocorrelation function of N(t) is  RN()=2NoB sinc(2B)cos(2fc).

- If we take samples N(iTs) at any arbitrary sampling period Ts, explain whether the samples will in general be correlated or uncorrelated, (and say why?).

- Find a specific sampling rate fs=1/Ts that can make the samples N(iTs) uncorrelated? In this case, are the samples also independent (why?)

Hint: use the autocorrelation function!

Problem #2 (10pts)

Consider the AM signal s(t) = Ac[1 + kam(t)] cos(2fct) demodulated with envelope detection

It is known that, for strong signal assumption, the output SNR is given by: , where P and W are the power and bandwidth of the message m(t), and No/2 is the noise power spectral density.

- Find SNRc, and show that the receiver figure-of-merit is:

- Consider the case of tone modulation where the message m(t)=Am cos(2fmt). Find the figure of merit, and express it in terms of the modulation index =kaAm.

- Find the maximum value of the figure of merit (for part b)), and compare this to the case of DSB-SC with coherent demodulation. What is your conclusion?

Problem #3 (10pts)

  - Explain how FM can allow the communication system designer to obtain a tradeoff between SNR performance and bandwidth consumption

  - Compare FM and AM/DSB with respect to the above factor, and justify your answers.