KING FAHD UNIVERSITY OF PETROLEUM & MINERALS

DEPARTMENT OF ELECTRICAL ENGINEERING

EE 417:   Communications Engineering II

Major Exam II

| # NAME |  |
| --- | --- |
| # ID |  |

| ##### Question | ##### Score |
| --- | --- |

| 1 |  |
| --- | --- |
| 2 |  |
| 3 |  |

| ##### TOTAL |  |
| --- | --- |

Problem #1 (10pts)

An analog signal m(t) with amplitude between [-mmax,+mmax] is sampled, quantized, and encoded into a binary PCM.

- Assuming uniform quantization with R bits per sample, show that the quantization noise power is given by , and the resulting signal-to-quantization noise ratio is  where P is the signal power.

- Suppose we use DPCM instead of PCM, and the resulting error signal (to be quantized) has a dynamic range [-dmax,+dmax], where dmax = mmax/4. If we use the same R as in PCM, what is the impact on the SQNR (give an exact answer)?

- For the DPCM case, if we want to fix the same SQNR as with PCM, what will be the change in R (#bits/sample)?

Problem #2 (10pts)

Consider a band-limited channel with bandwidth [-1MHz,+1MHz] used to transmit digital binary symbols at a rate R bps

- Assume the transmitter uses a pulse g(t), the channel response is c(t), and the receiver uses a filter h(t). What is the condition for ISI-free transmission on this channel? (express this for the overall system response p(t) time-domain and P(f) in frequency-domain)

- Give the maximum possible transmission rate over this channel (with zero-ISI). Indicate what type of p(t)&P(f) can achieve this maximum rate

- Since the ideal pulse response is not implementable, we want to use a raised –cosine spectrum-shaping pulse with a roll-off factor (excess-bandwidth) of 50%. What will be the new maximum achievable bit rate (without ISI) in this case?

Problem #3 (10pts)

A digital baseband signal with +/-A amplitude is transmitted using a rectangular unit pulse g(t) of duration T over an AWGN channel with PSD No/2, and detected using a matched filter receiver.

- Sketch the impulse response of the matched filter, and the matched filter output (when the input is g(t)).

- Verify that the optimum sampling time (to maximize the matched filter signal output) is at t = T, for the first bit (nT, for the n-th bit)

- If there is sampling time error (due to poor synchronization) with the receiver sampling time shifting to to= 0.9T, determine the SNR loss (in dB)

- Consider a 2nd system using a half-sine shaping pulse f(t)=  sin( t/T) over [0,T]. What should be the value of to guarantee that both systems will have the same error performance?  Give one possible advantage for using this 2nd system (with the sinusoidal pulse).