KING FAHD UNIVERSITY OF PETROLEUM & MINERALS

DEPARTMENT OF ELECTRICAL ENGINEERING

COURSE		:	EE -550; Introduction to Modern Control

EXAM			:	Midterm

DATE			:	April 18, 1995

PLACE		:	B. 14-102

TIME			:	8:00 - 9:30 PM

INSTRUCTOR	:	Dr. Samir A. AL-Baiyat

Name:

ID. Number

| Problem # 1 | /15 |
| --- | --- |
| Problem # 2 | /15 |
| Problem # 3 | /15 |
| Problem # 4 | /10 |
| Problem # 5 | /20 |
| Problem # 6 | /25 |
| Total | /100 |

Problem 1:

An ordered basis for the vector space (Â4[x], Â), where Â4[x] is the set of all real polynomials of degree less than 4 is B={1, x, x2, x3}. Consider the linear map j: Â4[x]® Â4[x] defined by

j(f(x)) = D2(f(x)) - 4D(f(x)) + f(x)  		for f(x)Î Â4[x]

where D2(f(x)) and D(f(x)) denote the second and first derivatives of the polynomial f(x) respectively. Find the representation of the linear map j with respect to the given base.

Problem 2:

If A is an n ´ n nilpotent matrix (i.e., Ak = 0 for some k > 0), then prove that zero is the only eigenvalue.

Problem 3:

Find the solution of

x(k+1) = A x(k),		y(k) = C x(k)

where the n ´ n matrix A is given by

A=11...1111...11..............11...11

and the 1  ´  n  matrix C

C=100...0

with x1(0) = x2(0) = x3(0) = ... = xn(0) .

Problem 4:

Find the state transition matrix f(t, to) of

x(t)·= e-At B eAt x(t)

What happen if AB=BA?

Problem 5:

Let P be a nonsingular matrix such that

PB=B10		PAP−1=A11A12A21A22

where B1  is an n1 ´ p matrix and rB = rB1 = n1. The matrices A21 and A22 have dimensions (n - n1) ´ n1 and (n - n1) ´ (n - n1), respectively. Show that {A, B} is controllable if and only if {A22, A21} is controllable.

Problem 6:

Approximate equations of motion for a hot air balloon (see the figure) are

where q = temperature change of air in balloon away from equilibrium temperature, u is proportional to change in heat added to air in balloon (control), v = vertical velocity, h = change in altitude from equilibrium altitude, and w = vertical wind velocity (disturbance).

1- Determine the transfer function from u to h and from w to  h. Is the system completely controllable by u? Is the system completely controllable by w?

2- Can the temperature  change q and a constant wind velocity w  be observed by a continuous measurement of altitude change h?(Assume, w  is a state and  u is the control.)