King Fahd University of Petroleum and Minerals

Physics Department

Phys. 203.02

May, 13th 1998

Quiz # 5

Problem #1

A Si  p-n junction has a doping of   Na = Nd=1014cm-3.

- Indicate on the figure the p-side and the  n-side of the junction.

- Calculate the barrier height at room temperature, (  ni=1.45  1010cm-3).

- Give the direction of the electric field and sketch its variation within the depletion region. What is its maximum value? Given the dielectric constant Si=1.05210-10  SI

Problem #2 Capacitance versus Voltage C(V) measurements performed on a Germanium made p-n junction gave the results reported on the figure below.

- What is the built-in voltage of the diode?

- What is the dopant concentration? Given the dielectric constant Ge =1.41510-10  SI